DEFINE

Abbreviation	DF

Arguments	Одно или более имен разделенные запятыми.

Default	None.

Description	Директива DEFINE определяет предопределенные имена для вставляемых строк, которые могут быть определены препроцессором с директивами #if, #ifdef, и #ifndef.  Предопределенные имена копируются точно так, как они описаны.  Команда чувствительна к регистру.

Example

DISABLE

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | None. |

Description	Директива DISABLE указывает компилятору генерировать код, запрещающий все прерывания для описываемой функции.  DISABLE должна определяться с #pragma непосредственно перед описанием функции. DISABLE применяется только к одной функции и должна переопределяться для каждой новой функции.

# NOTES

DISABLE может быть задана только с использованием директивы #pragma.

DISABLE может определяться не единожды в исходном файле для каждой функции которая используется с запретом прерываний.

Функция с запретом прерываний не может возвращать битовое значение.

Example

Пример функции использующей директиву DISABLE.  Заметим, что регистр EA очищается перед началом выполнения функции (JBC EA,?C0002) и восстанавливается перед выходом (MOV EA,C).

stmt level    source

1          typedef unsigned char  uchar;

2

3          #pragma disable      /* Disable Interrupts */

4          uchar dfunc (uchar p1, uchar p2)  {

5   1        return (p1 * p2 + p2 * p1);

6   1      }

; FUNCTION _dfunc (BEGIN)

0000 D3		SETB	C

0001 10AF01		JBC	EA,?C0002

0004 C3		CLR	C

0005	?C0002:

0005 C0D0		PUSH    PSW

;---- Variable 'p1' assigned to register 'R7' ----

;---- Variable 'p2' assigned to register 'R5' ----

; SOURCE LINE # 4

; SOURCE LINE # 5

0007 ED		MOV	A,R5

0008 8FF0		MOV	B,R7

000A A4		MUL	AB

000B 25E0		ADD	A,ACC

000D FF		MOV	R7,A

; SOURCE LINE # 6

000E	?C0001:

000E D0D0		POP	PSW

0010 92AF		MOV	EA,C

0012 22		RET

; FUNCTION _dfunc (END)

## NOEXTEND

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | EXTEND |

Description	NOEXTEND control задает использование только конструкций языка ANSI C.  Расширенния языка C51 исключаются.  Зарезервированные слова bit, reentrant, не распознаются и вызывают ошибку или предупреждение.

Example

#pragma NOEXTEND

## RESTORE

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | None. |

Description	The RESTORE control option fetches the values of the last SAVE directive from the save stack.

NOTE	RESTORE may be specified only as an argument of a #pragma statement.

See Also	SAVE

Example

#pragma save

#pragma noregparms

extern void test1 (char c, int i);

extern char test2 (long l, float f);

#pragma restore

Parameter passing in registers is disabled for the two external functions test1 and test2.  Afterwards, the settings at the time of the SAVE directive are restored.

SAVE

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | None. |

Description	The SAVE directive stores the current settings of AREGS and REGPARMS, and the current OPTIMIZE level and emphasis.  These settings are saved before an #INCLUDE directive and restored (using the RESTORE directive) afterwards.

Максимальная глубина вложения директивы SAVE восемь уровней.

NOTE

SAVE можетбыть определена только как аргумент #pragma.

See Also

RESTORE

Example

#pragma save

#pragma noregparms

extern void test1 (char c, int i);

extern char test2 (long l, float f);

#pragma restore

Parameter passing in registers is disabled for the two external functions, test1 and test2.  The settings at the time of the SAVE directive are restored by the RESTORE directive.

## ASM

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | None. |

Description	The ASM directive signals the beginning merge of a block of source text into the  .SRC  file generated using the SRC directive.

This source text can be thought of as in-line assembly.  However, it is output to the source file generated only when using the SRC directive.  The source text is not assembled and output to the object file.

The ENDASM directive is used to signal the end of the source text block.

NOTE

The ASM directive can occur only in the source file as part of a #pragma directive.

Смотри также	ENDASM

Example

#pragma asm

The following C source file:

.

.

.

stmt level	source

1	extern void test ();

2

3	main ()  {

4   1	  test ();

5   1

6   1	#pragma asm

7   1	  JMP   $  ; endless loop

8   1	#pragma endasm

9   1	}

сгенерируется файл  .SRC.

; ASM.SRC generated from: ASM.C

NAME	ASM

?PR?main?ASM			SEGMENT CODE

EXTRN  CODE (test)

EXTRN  CODE (?C_STARTUP)

PUBLIC main

; extern void test ();

;

; main ()  {

RSEG  ?PR?main?ASM

USING 0

main:

; SOURCE LINE # 3

;   test ();

; SOURCE LINE # 4

LCALL test

;

; #pragma asm

JMP  $    ; endless loop

; #pragma endasm

; }

; SOURCE LINE # 9

RET			; END OF main

END

CODE

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| CD | None. | NOCODE |

Description	Директива CODE добавляет ассемблерные команды в листинг.  The assembler code is represented for each function contained in the source program.  По умолчанию ассемблерный код не добавляется в листинг.

Example

#pragma code

Следующий пример представляет программу на C и скомпилированный объектный код и его мнемонику.  Номера строк исходной программы добавляются к строкам ассемблера. Символы R  и  E  означают Relocatable и External, соответственно.

stmt level	source

1	extern unsigned char  a, b;

2	       unsigned char     c;

3

4	main()

5	{

6   1	  c = 14 + 15 * ((b / c) + 252);

7   1	}

.

.

.

ASSEMBLY LISTING OF GENERATED OBJECT CODE

; FUNCTION main (BEGIN)

; SOURCE LINE # 5

; SOURCE LINE # 6

0000 E500	E	MOV	A,b

0002 8500F0	R	MOV	B,c

0005 84		DIV	AB

0006 75F00F		MOV	B,#0FH

0009 A4		MUL	AB

000A 24D2		ADD	A,#0D2H

000C F500	R	MOV	c,A

; SOURCE LINE # 7

000E 22		RET

; FUNCTION main (END)

AREGS

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| None. | None. | AREGS |

Description	Директива AREGS задает компиллятору использование абсолютной адресации регрстров с R0 по R7.  Абсолютная адресация улучшает быстродейстаие работы программы.  Например, команды PUSH и POP работают только с прямой или абсолютной адресацией.  Используя директиву AREGS, функции могут непосредственно сохранять и восстанавливать регистры.

Можно использовать директиву REGISTERBANK для определения используемого банка регистров.

NOTE

Though it may defined several times in a program, the AREGS option is valid only when defined outside of a function declaration.

Смотри также	NOAREGS

Example

Пример показывает использование обеих директив NOAREGS и AREGS.

stmt level    source

1          extern char func ();

2          char k;

3

4          #pragma NOAREGS

5          noaregfunc ()  {

6   1        k = func () + func ();

7   1      }

8

9          #pragma AREGS

10          aregfunc ()  {

11   1        k = func () + func ();

12   1      }

.

.

.

; FUNCTION noaregfunc (BEGIN)

; SOURCE LINE # 6

0000 120000	E	LCALL	func

0003 EF		MOV	A,R7

0004 C0E0		PUSH	ACC

0006 120000	E	LCALL	func

0009 D0E0		POP	ACC

000B 2F		ADD	A,R7

000C F500	R	MOV	k,A

; SOURCE LINE # 7

000E 22		RET

; FUNCTION noaregfunc (END)

; FUNCTION aregfunc (BEGIN)

; SOURCE LINE # 11

0000 120000	E	LCALL	func

0003 C007		PUSH	AR7

0005 120000	E	LCALL	func

0008 D0E0		POP	ACC

000A 2F		ADD	A,R7

000B F500	R	MOV	k,A

; SOURCE LINE # 12

000D 22		RET

; FUNCTION aregfunc (END)

.

.

.

Обратите внимание на различае в сохранении регистр R7 в стеке.  Код генерирутся для функции noaregfunc:

MOV  A,R7

PUSH ACC

Тогда как для функции aregfunc:

PUSH AR7

COMPACT

| Abbreviation | Arguments | Default |
| --- | --- | --- |
| CP | None. | ### SMALL |

Description	This directive controls the selection of the memory model to be implemented.

В модели памяти COMPACT, все переменные и локальные сегменты данных функций и процедур располагаются во внешней памяти данных 8051 system.  This external data memory may be up to 256 bytes (one page) long.  With this model the short form of addressing the external data memory through @R0/R1 is used.

Regardless of memory model, variables may be declared to be in any of the 8051 memory ranges.  However, place frequently used variables (such as loop counters and array indices) in internal data memory.  This significantly improves system performance.

NOTE

The stack required for function calls is always placed in internal memory.

See Also

SMALL, LARGE, ROM

Example

#pragma compact

DEBUG

Abbreviation	DB

Arguments	None.

Default	NODEBUG

Description	The DEBUG directive instructs the compiler to include debugging information in the object file.  By default, debugging information is excluded from the generated object file.

Debug information is necessary for the symbolic testing of programs.  This information contains both global and local variable definitions and their addresses, as well as function names and their line numbers.  Debug information contained in each object module remains valid through the L51 Link/Locate procedure.  This information can be used by DS51 or any of the Intel-compatible emulators.

NOTE

The OBJECTEXTEND directive can be used to instruct the compiler to include additional variable type definition information in the object file.

See Also

OBJECTEXTEND

Example

#pragma db

INTERVAL

Abbreviation	None.

Arguments	An optional interval, in parentheses, for the interrupt vector table.

Default	INTERVAL (8)

Description	The INTERVAL directive specifies an interval for interrupt vectors.  The interval specification is required for SICO-51 derivatives which define interrupt vectors in 3-byte intervals.  Using this directive, the compiler will locate vectors at absolute address (interval Ч n) + offset + 3,

where:

n	 is the interrupt number.

interval	 is the argument of the INTERVAL control directive (default 8).

offset	 is the argument of the INTVECTOR directive (default 0).

See Also

INTVECTOR, NOINTVECTOR

Example

#pragma interval(3)

INTPROMOTE

Abbreviation	IP

Arguments	None.

Default	INTPROMOTE

Description	The INTPROMOTE directive enables ANSI integer promotion rules.  Expressions used in if statements are promoted from smaller types to integer expressions before comparison.  This allows Microsoft C and Borland C programs to be ported to C51 with fewer modifications.

Because the 8051 is an 8-bit processor, use of the INTPROMOTE directive generates inefficient code in some applications.

See Also

NOINTPROMOTE

Example

#pragma intpromote

The following example demonstrates code generated using INTPROMOTE, and with NOINTPROMOTE.

stmt level	source

1	char c;

2	unsigned char  c1,c2;

3	int  i;

4

5	main ()  {

6    1	 if (c == 0xff) c = 0;	/* never  true  in  C51 V3.40 */

7    1	 if (c == -1) c = 1;	/* works in both versions */

8    1	 i = c + 5;

9    1	 if (c1 < c2 +4) c1 = 0;

10    1	 }

.

.

.

Code generated by C51 V4 using INTPROMOTE

; FUNCTION main (BEGIN)

; SOURCE LINE # 6

0000 AF00		MOV  R7,c

0002 EF		MOV  A,R7

0003 33		RLC  A

0004 95E0		SUBB A,ACC

0006 FE		MOV  R6,A

0007 EF		MOV  A,R7

0008 F4		CPL  A

0009 4E		ORL  A,R6

000A 7002		JNZ  ?C0001

000C F500		MOV  c,A

000E	?C0001:

; SOURCE LINE # 7

000E E500		MOV  A,c

0010 B4FF03		CJNE A,#0FFH,?C0002

0013 750001		MOV  c,#01H

0016	?C0002:

; SOURCE LINE # 8

0016 AF00		MOV  R7,c

0018 EF		MOV  A,R7

0019 33		RLC  A

001A 95E0		SUBB A,ACC

001C FE		MOV  R6,A

001D EF		MOV  A,R7

001E 2405		ADD  A,#05H

0020 F500		MOV  i+01H,A

0022 E4		CLR  A

0023 3E		ADDC A,R6

0024 F500		MOV  i,A

; SOURCE LINE # 9

0026 E500		MOV  A,c2

0028 2404		ADD  A,#04H

002A FF		MOV  R7,A

002B E4		CLR  A

002C 33		RLC  A

002D FE		MOV  R6,A

002E C3		CLR  C

002F E500		MOV  A,c1

0031 9F		SUBB A,R7

0032 EE		MOV  A,R6

0033 6480		XRL  A,#080H

0035 F8		MOV  R0,A

0036 7480		MOV  A,#080H

0038 98		SUBB A,R0

0039 5003		JNC  ?C0004

003B E4		CLR  A

003C F500		MOV  c1,A

; SOURCE LINE # 10

003E	?C0004:

003E 22		RET

; FUNCTION main (END)

CODE SIZE = 63 Bytes

Code generated by C51 V4 using NOINTPROMOTE

; FUNCTION main (BEGIN)

; SOURCE LINE # 6

0000 AF00		MOV  R7,c

0002 EF		MOV  A,R7

0003 33		RLC  A

0004 95E0		SUBB A,ACC

0006 FE		MOV  R6,A

0007 EF		MOV  A,R7

0008 F4		CPL  A

0009 4E		ORL  A,R6

000A 7002		JNZ  ?C0001

000C F500		MOV  c,A

000E	?C0001:

; SOURCE LINE # 7

000E E500		MOV  A,c

0010 B4FF03		CJNE A,#0FFH,?C0002

0013 750001		MOV  c,#01H

0016	?C0002:

; SOURCE LINE # 8

0016 E500		MOV  A,c

0018 2405		ADD  A,#05H

001A FF		MOV  R7,A

001B 33		RLC  A

001C 95E0		SUBB A,ACC

001E F500		MOV  i,A

0020 8F00		MOV  i+01H,R7

; SOURCE LINE # 9

0022 E500		MOV  A,c2

0024 2404		ADD  A,#04H

0026 FF		MOV  R7,A

0027 E500		MOV  A,c1

0029 C3		CLR  C

002A 9F		SUBB A,R7

002B 5003		JNC  ?C0004

002D E4		CLR  A

002E F500		MOV  c1,A

; SOURCE LINE # 10

0030	?C0004:

0030 22		RET

; FUNCTION main (END)

## INTVECTOR

Abbreviation	IV

Arguments	An optional offset, in parentheses, for the interrupt vector table.

Default	INTVECTOR (0)

Description	The INTVECTOR directive instructs the compiler to generate interrupt vectors for functions which require them.  An offset may be entered if the vector table starts at an address other than 0.  Using this directive, the compiler generates an interrupt vector entry using either an AJMP or LJMP instruction depending on the size of the program memory specified with the ROM directive.  Vectors are located at absolute address (interval Ч n) + offset + 3,

where:

n	 is the interrupt number.

interval	 is the argument of the INTERVAL directive (default:8).

offset	 is the argument of the INTVECTOR directive (default 0).

See Also

INTERVAL, NOINTVECTOR

Example

#pragma iv(0x8000)

## SRC

Abbreviation	None.

Arguments	Имя файла ( опция ). An optional filename in parentheses.

Default	None.

Description	Директива SRC используется для создания ассемблерного исходника вместо объектного файла.  Этот файл может быть проассемблирован с помощью A51 assembler.  Если имя файла не определена в параметре parentheses, имя файла и его путь берутся из исходного C файла, но он будет иметь расширение  .SRC.

NOTE	Компиллятор не может одновременно создать асемблерный исходник и объектный файл одновременно.

See Also	ASM, ENDASM

Example